Sparse Delta Integration (SDI)…

Esteban Rivera1, Charlie Démené1, Mickael Tanter1

1 Physics for Medicine Paris, INSERM U1273, ESPCI Paris, PSL University, CNRS, Paris, France

Correspondence: Mickael Tanter

Abstract

# Introduction

Accurate and reproducible simulation of ultrasound image formation is essential for designing transducers, testing beamforming strategies, and evaluating new imaging methods. Two broad classes of models are widely used. Full-wave solvers (for example, finite-difference and k-space pseudospectral methods) numerically solve the acoustic wave equations and can handle heterogeneous media and nonlinear propagation [1,2]., and on the other hand we have the Spatial impulse response (SIR) or linear-systems methods that assumes linear acoustics and compute fields by superposing elementary source responses from the transducer aperture making it in general fast and efficient.[3,4].

The SIR idea is rooted in classical results for baffled pistons (Tupholme; Stepanishen), which express transient radiation as an integral of geometry-dependent impulse responses convolved with the source excitation [5,6]. Building on this foundation, Jensen & Svendsen (1992) proposed a practical algorithm that discretizes the radiating surface into small rectangles and sums their time-shifted contributions; this approach underpins the Field II simulator [7].

Field II is a widely used MATLAB toolbox that implements Jensen’s SIR methodology and supports arbitrary transducer geometries, dynamic focusing, apodization, and pulse-echo simulations [7,8]. The Field II distribution and user guide describe implementation details and provide canonical examples for validation [8]. Because it is based on linear acoustics, Field II is computationally efficient for large apertures and long simulated times, but it does not model nonlinear propagation or complex multiple-scattering in heterogeneous media; such effects are handled by full-wave tools such as k-Wave [2,9].

So far, Field II is distributed from the author’s site as MATLAB source together with compiled components. A commercial parallel version, Field IIPro, is also offered, which includes a Python interface but is not open-source and requires licensing. As a result, there is currently no free Python version of Field II, limiting options for researchers who wish to modify or accelerate the low-level engine algorithm, or integrate the simulations to other python libraries [8].

Python has become a dominant language for scientific computing, machine learning, and data analysis. Its ecosystem drove the create of PyTorch, a fully featured framework for building deep learning models which provides an imperative, GPU-capable framework with automatic differentiation used for optimization via gradient descent, that could be use in inverse-problems research [10]. In addition, community projects such as Nilearn and the BrainGlobe initiative illustrate Python’s strong adoption in neuroimaging and computational neuroanatomy, providing several Python-based software tools to access, analyze and visualize anatomical data [11,12] that one could be interested to integrate with ultrasound simulations when working in neuroscience.

In this paper we present TorchField, an open-source, PyTorch-based implementation of the far-field Tupholme–Stepanishen SIR method. TorchField reformulates the trapezoidal impulse from a piecewise description to a double integration of Dirac delta distributions, allowing a vectorized tensor formulation suited for GPU execution and automatic differentiation in PyTorch. Key features are: (i) a fast, differentiable SIR engine with optional GPU acceleration; (ii) support for arbitrary patch-defined transducers; (iii) a reproducible, unit-tested codebase with examples that mirror Field II for validation; (iv) integration with gradient-based optimization workflows; and (v) compatibility with neuroanatomy datasets provided by projects such as BrainGlobe. TorchField is intended as a Python-native, open alternative for rapid prototyping and model-based research within the linear-acoustic regime, particularly in the neurosciences.

In addition, we introduce PyField, an open-source and freely available simulator that closely follows the Field II methodology. PyField provides a Python platform with transparent source code for calculating SIR and monochromatic pressure fields.

# Background Theory

This section describes the simulation method used with the far-field Tupholme–Stepanishen SIR [] method used in FIELDII [] and PyField, and how it could be then used to compute monochromatic pressure fields. The development presented here is based on [6], however, for the sake of completeness, a detailed development of the approach is presented.

## Basic theory

Consider the problem of determining the time-dependent pressure in the half-space z≥0, resulting from the velocity of the aperture. The aperture is assumed to vibrate with a specified normal velocity vn(r,t) which may be nonsinusoidal and/or pulsed, and the medium in the half-space is assumed to be isotropic with a constant velocity of propagation. In addition, the aperture delimited by the area S is assumed to be mounted in an infinite planar baffle, so that the normal velocity vanishes outside S {Fig. system showing the assumptions}.

The problem is formulated as a classical boundary value problem in the wave-equation of velocity potential Φr,t, which satisfies the homogeneous wave equation:

∇2Φr,t-1c02∂2Φr,t∂t2=0,  r∈B  [1]

Where c0 is the speed of sound in the medium, and B is the half-space z≥0 (B=x,y,z∈R3  z≥0} ).

The boundary condition of the normal velocity on the aperture surface and the system initial conditions yields to

∂Φ∂z=vnr,t,      r∈S, t>0  [2]

And

Φr,t​t=0=∂∂tΦr,t​t=0=0   [3]

The solution of this system can be obtained using a Green’s function representation. The velocity potential may then be expressed as

Φr,t=0tSvnra, t0gr,tra,t0) dS dt0   [4]

Where gr,tra,t0) is the time-dependent green’s function for the problem, vnr, t is the normal velocity, and the  ra denotes source coordinates on the aperture. Using the method of images, the Green’s function for a baffled piston is given by [Morse & Ingard, Blackstock]:

gr,tra,t0=δ(t-t0-r-ra/c0) 2πr-ra    [5]

Substituting Eq 5 intro Eq. 6, gives

Φr,t=0tvnt0S δ(t-t0-r-ra/c0) 2πr-ra dS dt0[6]

Where since the aperture is rigid the velocity is independent of the spatial position on the aperture, i.e. vnr,t=vnt for r∈S. We now define the Spatial Impulse Response (SIR) as

hr,t=S δ(t-r-ra/c) 2πr-ra dS [7]

With this definition, Eq. (7) becomes the convolution

Φr,t=0tvnt0hr,t-t0 dt0=vnt*hr,t [8]

Where * denotes the convolution in time. From the velocity potential, the acoustic pressure field is obtained as

pr,t=ρ0∂Φr,t∂t [7]

Which yields

pr,t=ρ0∂vnt∂t*hr,t [8]

Thus, the pressure field is computed as the convolution of the SIR - which encodes the spatial contribution or geometric features of the aperture - with the velocity vnt​​, which encodes the temporal information of the excitation.

## The Far-Field SIR for a rectangular aperture

In the previous section, we derived how the pressure field can be computed from the Spatial Impulse Response (SIR), following the classical Tupholme–Stepanishen formulation that underlies Field II [ref]. Equation (7), which defines the SIR, is a direct expression of Huygens’ principle: the acoustic field at a point  r is obtained by summing the spherical wavelets radiated from each infinitesimal element of the aperture surface, evaluated at the appropriate retarded time.

This formulation can be recast using acoustic reciprocity. Instead of viewing the aperture as the emitter of secondary sources, one can equivalently consider a spherical wave originating at the field point and propagating backward toward the aperture. The problem then reduces finding the intersection of this backward-projected spherical wave with the aperture surface. Geometrically, the field point is projected onto the aperture plane, and the expanding spherical wave appears as a circle whose overlap with the aperture determines the contribution to the SIR. For instance, rewriting the integral in polar coordinates leads to the alternative formulation [Jensen]:

hr,t=θ1θ2d1d2δ(t-R/c0)2πRrdrdθ  [12]

where r is the projected radial coordinate, R2=r2+zp2 is the distance from the field point to the aperture, and zp is the field point height above the aperture plane. The integration limits d1, d2, and θ1,θ2 are set by the aperture boundaries.

NOT SURE IF SOMETHING LIKE THIS. STILL NEED TO ADD THE CORRESPONDING SIR SHAPE.

For rectangular apertures, an exact solution to this integral has been derived in [6]. To build intuition, it is helpful to consider the problem geometrically. The impulse response at a given time corresponds to the fraction of the aperture that contributes coherently at that instant. Thus, the SIR can be understood in terms of isodistance curves from the field point. In the near field, these curves appear as rounded contours [Fig. circle contours], whereas in the far field, they approximate straight lines [Fig. Straight lines].

In general, under the assumption of the far-field, the SIR of a rectangular aperture tends to a trapezoidal shape [Fig. trapezoid]. This shape can be constructed by convolving two rectangular pulses whose widths correspond to the projections of the aperture’s length and width onto the line connecting the aperture center to the field point. Consequently, the trapezoidal SIR can be fully described by five parameters:

- t1​: time-of-flight (TOF) from the nearest aperture corner,

- t2, t3​: TOFs from the second and third nearest corners, which define the onset and end of the plateau,

- t4​: TOF from the farthest corner,

- hmax​: the plateau height,

Placing the rectangular aperture m of width and height of wmx,wmy in the x-y plane with its center at the coordinate rm, the field point position vector -coming from the m-th rectangle to the p-th field point- is   rm,p=rp-rm, where rp are the coordinates of the field point. This vector can be further decomposed into a distance lm,p=rm,p and an unit vector  um,p=rm,p rm,p=(xu,yu,zu)​. Using this description, one can get the following equations

t1=lm,pc0-Δt1+Δt22

t2=t1+Δt1

t3=t1+Δt2

t4=t1+Δt1+Δt2   [13]

Where Δt1 and Δt2, correspond to the min and max TOF for the side lengths of the rectangle

Δt1=minwmy yec0,wmx xec0  [14]

Δt2=maxwmy yec0,wmx xec0  [15]

Note that t2-t1=t4-t3=Δt1 and t3-t2=Δt2-Δt1, and that there are some cases where some of the t’s are identical and thus have Δt1 and/or Δt2 equal to zero.

The shape of the trapezoid is a function of Δt1 and Δt2 and so is its amplitude, but the area of the trapezoid integrated over the whole-time interval is always equal to [Jensen]

arecl=wmx wmy2πlm,p  [16]

Geometrically, the area can also be expressed as

arec=hmaxΔt2 [17]

Combining (16) and (17) gives the trapezoid amplitude

hm,pplateau=wmx wmy2πΔt2lm,p [18]

Therefore, the far-field trapezoidal SIR for a m-th rectangular aperture is

hmrp,t=hum,p,lm,p,wmx,wmy,t=sm,pt-t1,  t1<t<t2hm,pplateau,  t2<t<t3sm,pt4-t,  t3<t<t40,  elsewhere   [19]

With

sm,p=hm,pplateauΔt1 [20]

# The transducer’s Spatial Impulse Response.

## Naïve method:

When simulating arbitrary transducer geometries, it is rarely guaranteed that an entire piezoelectric element will satisfy the far-field condition. To resolve this, each element is subdivided into smaller rectangular patches. Each subdivision is assumed to operate in the far-field regime, and the total aperture response is then obtained by summing contributions from all subdivisions.

To ensure validity, the patch dimensions must satisfy [Jensen]:

w≪4lc0f  [21]

Where l is the distance to the field point, w the largest dimension of the rectangle and f is the highest frequency in the response simulated.

Moreover, real ultrasound transducers rarely behave like ideal pistons. Their vibration profile may be modified by boundary conditions at the edges or intentionally shaped to suppress sidelobes. For instance, applying a Gaussian apodization reduces sidelobes by weighing the active aperture [CITE]. Apodization can be incorporated into Eq. (19) by scaling the SIR of each element with a factor ae​, where ae=1 denotes a fully active element and ae=0 an inactive one. Similarly, delays can be introduced as a temporal translation of the SIR and can be further used to steer or focus the beam.

SOMETHING SIMILAR TO THIS

Therefore, the general expression for an ultrasound transducer composed by M rectangular patches is,

htxrp, t≈m=1Mamhmrp,t-τm[22]

Where hm was previously defined in eq[19]. Note that for an element e of this transducer, there is a subset of patches m such that am=ae and τm=τe. That is to say, the patches over an element will have the same value of apodization and delay. Finally, the overall impulse response of a transducer with E elements, each divided into M subdivisions, is:

htxrp, t≈eEherp, t=eEmeMaehe,m(rp,t-τe)  [23]

An example of the influence of transducer geometry, and elements apodization and delays can be seen in {FIG. TRANSDUCER WITH DELAYS AND APODIZATION}.

## A new approach: Sparse Delta Integration (SDI).

From the derivation on the equation [19], we can see that the first derivative of the trapezoid piecewise function can be written as follows
∂hmrp,t∂t=sm,p,  t1<t<t2-sm,p,  t3<t<t40,  elsewhere

=sm,p∙ut-t1-ut-t2-ut-t3+ut-t4   [24]

Where sm,p is function of the m-patch geometry and the point field p as depicted in equation 20, and u(t) is the Heaviside step function, defined as

ut=1,  t≥00,  t<0

Figure X. a) Simplified representation of a linear transducer geometry without elevation focusing or element subdivisions, showing transducer dimensions and element numbering. b) Element-wise delay profile (colored by apodization weights) for the default case (no delays, no apodization). c) Same as (b), but with a delay profile focused at (3, 0, 8) mm and a Hamming apodization with F/D = 1. d) Trapezoidal impulse responses at the focal point (3, 0, 8) mm for each element, color-coded by element index, using the delays and apodization from (b). e) Same as (d) but using the delays and apodization from (c). f) Summed response of all elements at (3, 0, 8) mm, yielding the spatial impulse response (SIR) corresponding to the configuration in (b). g) Same as (f), but for the configuration in (c).

If we continue with further derivation, we can get to the next expression

∂2hmrp,t∂t2=sm,p∙δt-t1-δt-t2-δt-t3+δt-t4=sm,pΔδt   [25]

Where δt stands for the unit impulse or unit Dirac delta distribution, defined as

δt=1,  t=00,  t≠0

Thus, we can rewrite the piecewise function shown in [19] as

he,mrp,t=-∞t-∞t'sm,pΔδt''dt''dt'  [26]

Where,

Δδt=δt-t1-δt-t2-δt-t3+δt-t4  [27]

Thanks to the additive property of the integrals, the SIR for a transducer composed by M patches become,

htxrp, t≈mMamhe,m(rp,t-τm)=-∞t-∞tmMamsm,pΔδt-τmdtdt  [28]

## Comparison between methods:

So far, the derivation has been expressed in terms of continuous time. For implementation, however, we must discretize the temporal axis. The continuous variable t is replaced by its sampled version tk, a vector of T samples defined as:

tk=t0+kΔt, k=0, 1, 2,…T-1.  [24]

with sampling step Δt=1/fsampling, where fsampling is the sampling frequency, and t0 is the initial time.

The number of trapezoids to be accumulated over this temporal grid is equal to the combinations of field points and patches on the transducer. In the naive method, each patch m is evaluated by filling the trapezoid following piecewise like function in equation [19] over all its nonzero time samples. Then, If on average the trapezoids span a number Δk of sample times out of a temporal grid with T samples, for P field points the computation time (CT) becomes of the order of

CTNaive~OP∙M∙Δk   [29]

For clarity, consider the case of a single field point  P=1:

CTNaive(P=1)~OM∙Δk   [30]

In contrast, the Sparse Delta Method (SDM) proposed in Eq. [24] avoids filling entire trapezoids. Instead, each patch contributes only eight Dirac deltas (explanation of eight instead of the four deltas presented in [28] is developed in the next section), requiring CTmMamsm,pΔδt-τm~O8M, at the expense of adding the cost of the two integrations over time each with ~O(T). Hence,

CTSDM(P=1)~O8M+2T [31]

In general, the SDI algorithm will be faster if,

M∙Δk≫8M+2T   [32]

Which leads to

Δk≫8+2TM   [33]

Suppose we have a configuration of M patches computing the values at P points. The number of time samples is computed as follows

T=Δttotal fs

Δttotal=maxt4-mint1=lmax-lminc0+Δt1+Δt2max2+Δt1+Δt2min2

With,

Δt1+Δt2=wxuxc0+wyuyc0≈wc0ux+uy

For implementation the smallest value of Δt1,  Δt2 are one discrete step, dt=1/fs, and the biggest values of for the components of the unit vector are ux=uy=12, then maxux+uy=2<2, which gives us

2fs≤Δt1+Δt2<2wc0

Taking the overestimated limit, we get to

Δttotal≈lmax-lmin+wc0

In the simplest case where we have just one point,

TP=1 ~ wfsc0

And if we are computing the time samples for a field of points in the far field condition, lmin~w24λ, if we suppose that the farthest point meets this condition lmax≫w, then we have

T ~ lmaxfsc0

And we can compute the number of patches as follows

M=Awxwy≈Aw2

Therefore,

TM~ lmaxw2A∙fsc0

Taking the middle point in the interval 0,2wc0 as approximation of the mean trapezoid samples, we get to

Δk ~ 2wc02∙fs=wfsc0

From 33, we find that the SDI algorithm will be faster if,

wfsc0≫8+2lmaxw2Afsc0

Awfs≫8Ac0+2lmaxw2fs

Which leads to

A≫2w2fslmaxwfs-8c0

If w≫8 c0/fs, then

A≫2wlmax

The last equation shows that the SDI method will be faster when the transducer aperture is much higher than the larger subdivision length times the longest distance between patches and field points.

# Pyfield: Overview of the program

Building on the above formulation, we developed an open-source Python simulators: PyField. This program contains different modules for simulation monochromatic pressure fields of arbitrary transducer geometries made from small rectangular apertures. Since PyField works from

PyField reproduces the Field II method with transparent, open code suitable for teaching and validation. In the following subsections, we detail the logic underlying these implementations.

## H_sir:

Let’s assume we have a set of P field points, stored in a matrix RP∈RP×3 containing the 3 spatial coordinates, and a transducer composed by Eelements, each divided with M rectangular patches.

In PyField algorithm, we first compute the five trapezoid parameters (eq. 13 and 18) for every patch-point combination. Each of these combinations is referred to as an Event. The events are stored in a tensor C∈RP×M×5.

From the event tensor C, the smallest t1 value across all events sets the starting time t0 of the temporal grid, while the larges t4 defines its final time tf. The number of samples is then chosen to satisfy:

T≥tf-t0fsampling=Δtspanfsampling [23]

We now allocate the output tensor h∈RP×T, which will store the impulse response for each field point across time.

The PyField’s naïve algorithm proceeds as follows:

- Loop over field points p=1,…, P.

  - For each point, retrieve the trapezoid parameters of all patch–point events from tensor C.

- For each point, loop over events p, m=1,…,M:

  - Compute the time interval indices (kstart,kend) that correspond to the trapezoid boundaries on the discrete time grid.

- For each event, loop each event time interval p, m, k=kstart, kend:

  - Within this index range, evaluate the trapezoidal SIR contribution hmrp,tk using the piecewise definition (Eq. 19).

- Accumulate contributions in the temporal grid:

  - Add the computed trapezoid values into the output tensor:

hp, kstart:kend +=hmrp,tkstart:kend

Where b +=a means b=b+a. That is to say, b is reassigned with its previous value plus a. By repeating this process, the contributions of all patches are accumulated for each point, producing the final tensor h, which contains the SIR at every field point. As shown in Eq. (11), this SIR tensor can be then used to compute the pressure field distribution.

In the case of the Sparse Delta Integration, the continuous variable t is replaced by its sampled version tk, a vector of T samples as defined in Eq. [24]. Unlike the piecewise trapezoidal function defined by the four times (t1, t2, t3,t4), the reformulated method uses delta functions located exactly at these times. Since t1,…,t4 do not necessarily coincide with the discrete grid tk, their contributions must be distributed to the closest discrete samples.

Intuitively, this means that instead of placing a delta “between samples” (which would be lost on the grid), we split its wight between the two nearest time samples. This ensures that the total contribution is preserved while being consistent with discrete representation.

Formally, given a continuous time t'∈R, its index position in the sampled grid is,

kt'=t'-t0dt  [35]

Here kt'∈R, while k∈Z+ represents valid discrete indices. Thus, instead of assigning the delta exactly at t'. Its amplitude is distributed between the two nearest discrete grid points, using linear weight:

wft'=kt'-kt',  wct'=kt'-kt'=wft'+1    [36]

Where now kt'∈Z+  and kt'∈Z+, and corresponds to the floor and ceil value of the number kt' that can be now used as index. Then, for taking the lower and higher value between {tk} closest to t' we define

tf=tkt,  tc=tkt       [37]

Where tkt',   means that we are evaluating our time vector at the index kt', and the same for kt. With this interpolation, the discrete version of the deltas defined in equation [29] becomes

Δδtk=Δδtkceil+Δδtkfloor     [38]

With,

Δδtkfloor=wft1δtk-t1f-wft2δtk-t2f-wft3δtk-t3f+wft4δtk-t4f      [39]

Δδtkceil=wct1δt-t1c-wct2δt-t2c-wct3δt-t3c+wct4δt-t4c     [40]

- This weighting ensures that the delta contributions are properly mapped onto the discrete time grid, by adding four more deltas. A schematic illustration of this interpolation strategy is shown in Figure X (middle-point positioning).

## Psimulation:

By computing the temporal Fourier transformation of this expression, one gets to

Pr,ω=iωρ0VωHr,ω  [9]

Where Pr,ω=Fpr,t, Vr,ω=Fvt, and Hr,ω=F{hr,t}.

In the special case of a monochromatic excitation at the transducer’s central frequency fc ​, the pressure amplitude is

pfcr=Pr,ωc=2πfcρ0VωcHr,ωc  [10]

which shows explicitly that

pfcr∝Hr,ωc  [11]

Hence, in the monochromatic regime, the spatial pressure distribution is proportional to the Fourier transform of the SIR at the operating frequency.

## Transducer:

## Atlases:

# Pyfield validation with FIELDII

# Annexes

In the regime of M≫T, this inequality requires Δk≪1. In general, the sampling frequency is chosen as most trapezoids are well sampled, thus Δk≫1 so this means that in most cases the SDM is going to be faster than the naïve one. Conversely, for the naïve method to be competitive, one must have, T≫M.

For calculation, the range of time Δtspan for the SIR is compute as Δtspan=Tfs, where fs corresponds to the sampling frequency. To not lose any trapezoid event, the minimum range of time when no delay is applied, is

Δtspanmin=dmax-dmin/c0  [35]

Where dmax=maxrm,p and dmin=minrm,p refers to the maximum and minimum distance between the center of the patches and the field-points, and c0 stands for the speed of sound in the medium. Thus,

Δtspanmin>dmax

For simplicity, let’s consider a linear aperture of value A. To compute the SIR of this aperture, we must divide it in M patches of length w that fall in far-field, that is

A=Mw

Such that,

dmax≫w24λ⇒dmax≫A24M2λ

From [35] we get the next inequation

Δtspanmin>dmaxc0⇒Tfs>dmaxc0

And with [36] and [37] we find,

T>fsA24M2c0λ

array placed in the x-y plane centered in the axis, with an aperture of A (so spanned from -A2,A2) , and a set of points located on the space under the transducer until a maximum depth of zf, so xp,zp=x,z∈R2 | x∈A2,A2 ∧z∈0,zf. Under this configuration the maximum distance becomes

dmax=zf2+A2  [36]

This shows that for most practical apertures and imaging depths, TorchField will be faster than the naïve PyField-style approach.

##

1) Write M

A=LxLy→M≈Awxwy

On the far-field condition:

w2=wxwy ~ 4lminλ

→M~A4lminλ

2) Samples in each trapezoid :

Δkm,p=t4-t1∙fs=Δt1+Δt2∙fs

Δkm,p=wy∙yuc0+wx∙xuc0∙fs

Δkmin=0, Δkmax=22wx+wyc0∙fs

→Δk ~22wc0∙fs

3) The temporal samples

T=fs∙Δtspan

where,

Δtspan=tmax-tmin≈lmax-lmin+wc0+τmax

Δtspan~lmaxc0→T~fslmaxc0

From that

2lminλc0∙fs≫8+2fslmaxc0A4lminλ

→A≫8∙fslminlmaxλfs2lminλ-8c0

fs2lminλ-8c0>0→lmin≫32fcc0c0fs2

→A≫8∙fslminlmaxλfs2lminλ=4lmax2lminλ

- Treeby BE, Cox BT. k-Wave: MATLAB toolbox for the simulation and reconstruction of photoacoustic wave fields. J Biomed Opt. 2010;15(2):021314.

- Cohen JB, et al. (example ref) — (Replace with specific FD/FDTD or k-space pseudospectral reference as needed).

- Jensen JA. Calculation of pressure fields from arbitrarily shaped, apodized, and excited ultrasound transducers. IEEE Trans Ultrason Ferroelectr Freq Control. 1992;39(2):262–267.

- Jensen JA. A new calculation procedure for spatial impulse responses in ultrasound. J Acoust Soc Am. 1999;105(6):3266–3274.

- Tupholme GE. Generation of acoustic pulses by baffled plane pistons. Mathematika. 1969;16(2):209–224.

- Stepanishen PR. Transient radiation from pistons in an infinite planar baffle. J Acoust Soc Am. 1971;49(5B):1629–1638.

- Jensen JA, Svendsen NB. (1992) — same as [3]; Field II implementation described in Jensen J.A.: "FIELD: A program for simulating ultrasound systems" and user guide.

- Jensen JA. FIELD: A program for simulating ultrasound systems — User’s guide and project pages (Field II distribution).

- Treeby BE, Cox BT. (same as [1]) — full-wave vs SIR comparison discussions.

- Paszke A, et al. PyTorch: An imperative style, high-performance deep learning library. NeurIPS. 2019.

- Abraham A, et al. Nilearn: a Python module for fast and easy statistical learning on neuroimaging data. NeuroImage. 2014; (or project website/docs).

- Tyson AR, et al. BrainGlobe: an open source toolkit for computational neuroanatomy (project website/docs).

[9] A. D. Pierce. Acoustics, An Introduction to Physical Principles and Applications. Acoustical Society of America, New York, 1989.